# Camera PCB Design

# Project context

- Record stable HD footage throughout the flight

- Fit within the avionics bay

- Store video locally and stream live video to the ground station

- Triggered by an STM32 that will receive directions from the main flight computer using the CAN protocol

- STM32 triggers recording

- Runcam Split handles video capture and storage.

# Functional Requirements

### Video

- Record 720p–1080p video at 30 fps (60 fps optional).

- Use RunCam Split-style cameras with onboard H.264 encoding to MP4.

- Each camera saves video to a high-endurance microSD card (≥ 32 GB).

- Support two to three cameras controlled by the STM32.

- Provide an optional analog video output for live FPV feed.

### Control and Feedback

- STM32 sends start/stop triggers to each camera via GPIO or UART.

- Cameras provide status signals (idle, recording, fault) to the STM32.

- MCU can power-cycle each camera through dedicated load switches.

- MCU broadcasts camera status and events over the CAN bus (1 Mbps).

- Optional UART link for VTX channel and power control (SmartAudio or Tramp).

### Power System

- Operates from a 9–16 V DC input.

- Includes a buck converter to 5 V (≥ 3 A) and an LDO regulator to 3.3 V for logic.

- LC filters on each camera and VTX power rail to reduce noise.

- Reverse-polarity, surge, and ESD protection on the power input.

- Hold-up capacitance to prevent resets during short power dips.

### Processing and Communication

- Main MCU: STM32F405 or STM32F407 with bxCAN.

- CAN transceiver: TI SN65HVD230D / DR.

- Communicates with other avionics via Classical CAN 2.0B at 1 Mbps.

- Supports I²C or SPI sensors for temperature, voltage, and IMU data.

- Real-time clock for timestamps and synchronised flight event logging.

### Data and Logging

- Logs temperature, supply voltage, and system health every few seconds.

- Stores log data to onboard flash or SD card.

- Records key events such as liftoff, apogee, and recording start/stop with timestamps.

### Reliability and Safety

- Watchdog timer resets the MCU on a fault.

- Automatic recovery from camera or power faults.

- Designed to withstand 15 g vibration, 50 g shock, and –10 °C to +60 °C operation.

- Uses locking connectors and strain-relieved wiring.

- Includes TVS diodes and a resettable fuse on main power input.

### Live Video Feed (Optional)

- Supports 5.8 GHz analog VTX for real-time ground video.

- VTX is powered and controlled by the STM32.

- Target latency below 50 ms.

# Parts selection (Pending Availability)

- Microprocesser: STM32F405RG or STM32F407VGT6

  - bxCAN (Classical CAN 2.0A/B, up to 1 Mbps)

- Camera Board: 2 -3 RunCam Split HD

- Can Transevier: TI SN65HVD230D / DR? (This is just the first one that came up, so it can be changed)

- PCB Parts: TBD, as I will need more guidance and look at parts availability

- Video Transmitter: FPV drone transmitter (5.8ghz)

# Architecture